EE460 Spacecraft Project Report

Heeirthan Shanthan

For the final project in EE460, we were assigned options to choose from when designing the control law of the spacecraft. My choices included varying mass, ramp commands, sequential commands, sensor configuration 1 with sensed values, a PID controller, as well as a graphical user interface for my user interface. In this report, I will reflect on the choices I made and the overall outcome of the project.

Mass Properties:

For mass properties, I chose to account for varying mass as the mass properties of the plant varies as fuel is burned. I did not account for moment of inertia, as I simply chose to prioritize my time improving other aspects of the project. Using varying mass properties improved the performance of my spacecraft, while it did not take me a significant amount of time to implement in comparison to some of the other choices I made. The way I implemented this is that I obtained and integrated the mass flow rate, before subtracting the “lost” mass from the original mass.

Command Strategy:

For the command strategy of my thrusters, I chose to model my commands as ramp commands. This was a choice based on getting the most out of ease of implementation while having a smooth and more fuel-efficient strategy than commands such as bang-bang. The smoothness of my trajectory also made it easier for me to reach tolerances for each of the three axes. This command required that I defined a final time to reach the points, which had a great impact on the performance of the spacecraft in terms of fuel efficiency and ability to settle at the point quickly. Setting a long final time to reach a point only a short distance away would waste time given that the overall time was 120 seconds, while setting a short final time for a point further away would lower fuel efficiency, increase overshoot, and decrease the settling time of the spacecraft.

My solution to this problem was to make the final times as dynamic as I could in the limited time frame I had to improve my project. In my script, I calculated the distance between the current and previous point, and based on that distance I assigned the final times for the ramp command. This was done assuming the spacecraft would move in order from 1-4. I am aware this could have been done more dynamically, by retrieving the previous command in the same way I retrieved the initial position for each ramp command. This way, the order of points wouldn’t have mattered. However, given the limited time I had, I saw this as an option that was better than constant final times. I could also have derived the final position using an algorithm based on the distance, but that would have been too time consuming. Additionally, I gave the theta commands a shorter final time as these commands took shorter time to complete than the translational commands, which made my spacecraft more time-efficient in its movements.

If I were to make my choice again, I would have chosen the cubic-spline command, as the trajectory would follow the command more smoothly and would likely save me time and fuel. Initially, I thought it would be too time consuming to implement, but in hindsight I realized it would not have been a lot more demanding than the ramp command.

Regulator Architecture:

For my regulator architecture, I chose to implement a PID controller. From a previous course in controls, I knew that the tradeoff between performance improvement and cost of implementation would benefit me more than it would hurt me. This was due to the ability to control my settling time, overshoot, while also being able to minimize the steady-state error with the integral gain. The tolerances for competing some of the points were very small, where my PID controller helped make me meet small tolerances while settling quickly the biggest strength of my spacecraft. Looking back on the project's course, this was probably my best control law design selection.

Although I was very happy with my PID controller, it was challenging to tune the gains. My method of tuning was literally just experimenting with each of the three gains and analyzing my response until I was happy with the results. However, I did end up spending some additional time selecting generalized gains for all the levels. Once I had configured gains for the third level, I ended up not being able to make all the points for the fourth level. Once I managed to tune the controller to complete the fourth level, the same gains did not work for the third level. Unfortunately, I had to repeat this step to optimize the performance on the levels beyond level 4, but it ended up giving me results that I was satisfied with. What I learned from the PID tuning was how sensitive the system was to slight changes in the ratio between Kd and Kp, and I also learned that my final and most optimized gains were higher than the ones I started out with. As for Ki, I was able to keep it very low without increasing steady-state error, as I also wanted to minimize settling time and overshoot.

Sensor/Observer Configurations:

My choice of sensor/observer configurations was undoubtedly my biggest regret of this project. I decided to use sensor configuration 1, which provided sensors for position, velocity, and acceleration for each of the axes. I also chose to only use the sensed values. In all honesty, I made my decision based on ease of implementation, without realizing the difficulties it would bear towards the end of the project.

To be specific, I had significant issues with handling more than one sensor failure per axis at a time. I might have been able to deal with two sensor failures at once, position and velocity, but after several hours of not being able to solve a drift issue, I directed my focus to optimize my strengths of the system and attempting to reduce noise due to lack of time. If I had used an observer, I could have been able to use the error estimate from a Kalman filter in my observer or something similar to avoid using the failed sensor value. If I had to choose again, I would definitely have chosen sensor configuration 3 and used a full-order observer. That way, if one or two sensors broke for an axis, I would have another one to spare. I could have been able to account for three broken sensors by only using the observer values, which would have been practically impossible for my configuration to handle. Lack of redundancy was without doubt the main reason for the level failures I experienced for this project.

Sequential Commands:

I chose to use sequential commands for my command sequencing strategy. This was mainly because I thought it sounded fun to implement, but I ended up regretting this choice. Not only did it make my spacecraft complete each point slower than it would have with combined commands, but it was also time-consuming to apply to my control law. The main issue I had was getting the rotational command timer to execute as soon as the tolerances were met for the translational commands. This was a good example of how extra work doesn’t always result in a better-performing system.

There are some benefits to sequential commands in real-world application, such as if a camera is attached to a spacecraft and needs to be facing in a certain direction before the rotation is done. However, this was not the case for the application of this project, and if I were to choose again I would certainly have stuck with the combined commands I already had implemented before making the decision.

User Interface:

For my user interface, I chose to design a GUI using the appdesigner tool in MATLAB. The GUI consists of four buttons, each corresponding to movement to each point. The buttons are color coded in coherence with the colors of the points, making it easier for the user to know which button would relate to which point. I am happy with that choice, as it was easy to implement and I couldn’t find any reason to not keep my user interface simple.

Combination of Selections:

While looking back at the overall combination of my selections, I was particularly happy with the combination of ramp commands and PID controller, which improved the overall control of the spacecraft as mentioned above. Apart from my non-ideal choice of sensor configuration, I believe the combination of my selections worked fine. I do, however, wish that I implemented a PID+F(s) or PID+F so that I would be able to track my commands better, while not spending a whole lot of time adding the extra gain to my system. Additionally, as my sequential commands slowed the spacecraft, I had to try to keep my Kp as high as possible while also trying to keep my final time for the ramp command as short as possible with the purpose of reducing the time of completion. Although it worked out well in the end, it might have cost me fuel points.

My Level:

I chose my level to test the strength of my spacecraft, which was settling quickly within tight tolerances. Therefore, I made the tolerances of each point small, while reasonable. I deliberately set the points up so that there was a significant rotation per point, just out of curiosity to see how well my spacecraft could handle the high demands. I ended up completing the level, which I was surprised to see. I did not focus too much on anything beyond that, having only one sensor fail to current, and one thruster fail to minimum. Furthermore, I also had sensor noise and fuel burn on, while keeping full fuel in my tanks. I did not see any point in aggressively testing my sensor failure handling as I knew that would be a lost cause to try to fix.

Lessons Learned/Recommendations:

The biggest lesson I learned overall was to include long-term consequences in my decision-making processes. At that time, I might have selected the least tedious strategy with sensor configuration 1, but it ended up becoming a huge burden for my project. On the contrary, I decided to choose the more difficult option when with sequential commands, which did not benefit me at all for this project. Looking back, I wish I had set aside more time to analyze the long-term pros and cons of each choice, as this would have saved me time in the future and improved my performance. Conclusively, I was the happiest with selecting ramp and PID and regretted choosing sensor configuration 1 and sequential commands.